Design and Implementation of a 2-bit ALU using Arduino

Members & ID :

| Omar Mostafa Salah | 921240131 |
| --- | --- |
| Rawan Essam Ismail | 921240066 |
| Fares Salama Mohammed | 921240139 |
| Mariz Medhat Iskandar | 921240151 |
| Marina Milad Atef | 921240152 |

Supervised by: Dr. Samar Ashraf & Dr. Medhat Hossein

## 1. Introduction

An Arithmetic Logic Unit (ALU) is a fundamental digital circuit that performs arithmetic and logical operations. It serves as the computational core of any Central Processing Unit (CPU). This project demonstrates the design of a 2-bit ALU implemented using an Arduino Uno controller. The system processes two 2-bit binary inputs to execute various arithmetic and logic functions selected via a 3-bit control bus.

## 2. System Specifications

The system is designed to handle:

- Inputs: Two 2-bit numbers (A and B) and a 3-bit selection code (Op).

- Outputs: A 2-bit result (F_1, F_0) and a Carry/Borrow indicator.

- Control: A 3-bit operation code to choose between 6 different functions.

## 3. Hardware Components and Interfacing

Based on the Tinkercad simulation and circuit diagrams, the following hardware configuration was used:

### A. Input Section (DIP Switches)

A 10-position DIP switch (or sliders) is used to provide logic states:

- Pins 2, 3: Input A (A_0, A_1).

- Pins 4, 5: Input B (B_0, B_1).

- Pins 6, 7, 8: Selection lines (S_0, S_1, S_2).

### B. Processing Unit

The Arduino Uno acts as the processing brain. It reads the digital states from the input pins, processes them using software-defined logic (Sketch), and updates the output pins.

### C. Output Section (LED Indicators)

Three LEDs with current-limiting resistors are used for visualization:

- Green LEDs (F0, F1): Display the 2-bit result.

- Red LED (Carry): Indicates an arithmetic carry (in addition) or a borrow (in subtraction).

## 4. Selection Logic and Function Table

The ALU executes operations based on the 3-bit selection input ($Op$). The logic follows the table below:

| Op (Decimal) | S2 S1 S0 | Operation |  |
| --- | --- | --- | --- |
| 0 | 000 | AND |  |
| 1 | 001 | OR |  |
| 2 | 010 | XOR |  |
| 3 | 011 | NOT A |  |
| 4 | 100 | Addition |  |
| 5 | 101 | Subtraction |  |

## 5. Software Implementation (The Code)

- 1. Pin Definitions (Lines 1–7)

- This section tells the Arduino where each component is physically connected.

- Lines 1–5: const int defines fixed variables for the input pins. A_0, A_1, B_0, B_1 are the data inputs, and S_0, S_1, S_2 are the selection lines.

- Lines 6–7: Defines the output pins for the result bits (F_0, F_1) and the Carry LED.

### 2. Setup Function (Lines 9–17)

- The void setup() runs once when the Arduino is powered on.

- pin Mode(..., INPUT): Configures the switch pins to listen for incoming signals.

- pin Mode(..., OUTPUT): Configures the LED pins to send out electricity to light them up.

### 3. Data Acquisition (Lines 20–23)

- Inside void loop(), the Arduino constantly reads the switches.

- Bit-Shifting (<< 1): Since digital Read only gives a 0 or 1, we use bit-shifting to combine two bits into a single integer. For example, in int A, the first bit is read normally, and the second bit is shifted to the "tens" place in binary.

- int Op: Combines S_0, S_1, and S_2 into a single number (from 0 to 7) to determine which operation to perform.

### 4. The Logic "Brain" (Lines 28–44)

- The switch(Op) block is where the ALU performs the calculation:

- Case 0, 1, 2 (AND, OR, XOR): These perform bitwise logic between A and B.

- Case 3 (NOT A): Inverts the bits of A. The & 3 ensures the result stays within 2 bits (11_2 = 3_{10}).

- Case 4 (Addition): * result = A + B; adds the numbers.

  - carry = (result > 3) ? 1 : 0; If the sum is greater than 3, the Red LED (Carry) is triggered because the result "overflows" two bits.

- Case 5 (Subtraction):

  - carry = (A < B) ? 1 : 0; If A is smaller than B, a "Borrow" is needed, so the Red LED lights up.

  - if(result < 0) result += 4; This handles negative results to keep the binary display correct for 2-bit logic.

### 5. Output Stage (Lines 46–49)

- Finally, the software results are sent back to the hardware.

- digital Write(F0_pin, result & 1): Sends the first bit of the result to the Yellow LED.

- digital Write(F1_pin, (result >> 1) & 1): Shifts the result to send the second bit to the White LED.

- digital Write(Carry_pin, carry): Sends the carry/borrow status to the Red LED.

## 6. Circuit Simulation (Tinkercad Analysis)

The simulation provided in the images shows a clean breadboard layout:

- Pull-down Resistors: Used on the input switches to ensure stable logic '0' when the switch is open.

- Wiring: Blue wires represent input signals to the Arduino, while cyan/green wires represent the output signals to the LEDs.

- Power: The breadboard is powered by the Arduino's 5V and GND pins.

## 7. Conclusion

This project successfully integrates software logic with hardware interfacing to create a functional 2-bit ALU. Unlike traditional hardware-only ALUs (using chips like 74LS181), this Arduino-based approach allows for easier modifications and debugging through code, demonstrating the efficiency of "Software-Defined Hardware" in modern digital design